IEI iDP Introduction

iDP (IEI Internal DisplayPort)

## The iDP connector is provided on the IEI products. Through the IEI iDP converter cards, the iDP connector can support different display specifications, such as VGA, HDMI, DVI-D and LVDS.

iDP

Flexible Display Output Options

iDP converter cards with brackets

Easy Installation on the Chassis

| iDP Supported Models |  |  |  |
| --- | --- | --- | --- |
| HPCIE-C236 | IMBA-Q170-i2 | KINO-DH110 | WAFER-AL |
| HPCIE-Q710 | IMBA-H110 | KINO-AQ170 | WAFER-ULT3/ULT4 |
| SPCIE-C236 | IMBA-Q870-i2 | KINO-DH810 | WAFER-BTW2 |
| PCIE-Q170 | IMBA-C2260-i2 | KINO-DQM871-i1 | WAFER-BT-i1 |
| SPCIE-C2260-i2 | IMB-H110 | KINO-ABT-i2 | WAFER-ULT/ULT2-i1 |
| PCIE-Q870-i2 | IMB-Q870-i2 | NANO-AL/ALW2 | WAFER-KBN-i1 |
| IMBA-C2360-i2 | IMB-H810-i2 | NANO-ULT3 |  |

DisplayPort Expansion Board (for IEI iDP Connector), RoHS

DP-DP-R10

Features

- DisplayPort to DisplayPort output

- Resolution support up to 1920x1200

# Specifications

Display Input: 1 x IEI iDP connector Display Output: 1 x DisplayPort Temperature: -20°C ~ 60°C Humidity: 5 ~ 95% non-condensing

Dimensions: 50 mm x 50 mm

Dimensions (mm)

| 1 x DP-DP expansion board | 1 x QIG |
| --- | --- |
| 1 x IEI iDP connector cable, 450 mm length |  |

| Part No. | Description |
| --- | --- |
| DP-DP-R10 | DisplayPort expansion board (for IEI iDP connector), RoHS |

Packing List	Ordering Information

DisplayPort to HDMI 1.3a Converter Board (for IEI iDP connector), RoHS

DP-HDMI-R10

Features

- DisplayPort to HDMI 1.3a output

- Support up to 1920x1200 resolution

# Specifications

Display Input: IEI iDP connector Display Output: 1 x HDMI 1.3a Display IC: Parade - PS161 (DP to HDMI 1.3a)

Temperature: -10°C ~ 60°C Humidity: 5 ~ 95% non-condensing

Dimensions: 50 mm x 50 mm

Dimensions (mm)

| 1 x DP-HDMI converter board | 1 x QIG |
| --- | --- |
| 1 x Internal DisplayPort cable, 450 mm length |  |

| Part No. | Description |
| --- | --- |
| DP-HDMI-R10 | DisplayPort to HDMI 1.3a converter board (for IEI iDP connector), RoHS |

Packing List	Ordering Information

DisplayPort to 24-bit Dual-channel LVDS Converter Board (for IEI iDP Connector), RoHS

DP-LVDS-R10

Features

- DisplayPort to LVDS output

- Supports 24-bit dual-channel up to 1920x1200

# Specifications

Display Input: 1 x IEI iDP connector

Display Output: 1 x LVDS 24-bit dual-channel Display IC: Chrontel - CH7511B (DP to LVDS) Temperature: -10°C ~ 60°C

Humidity: 5% ~ 95 non-condensing

Dimensions: 60 mm x 50 mm

Dimensions (mm)

| 1 x DP-LVDS converter board | 1 x QIG |
| --- | --- |
| 1 x Internal DisplayPort cable, 450 mm length |  |

| Part No. | Description |
| --- | --- |
| DP-LVDS-R10 | DisplayPort to 24-bit dual-channel LVDS converter board (for IEI iDP connector), RoHS |

Packing List	Ordering Information

DisplayPort to VGA Converter Board (for IEI iDP Connector), RoHS

DP-VGA-R10

Features

- DisplayPort to VGA output

- Support up to 1920x1200 resolution

# Specifications

Display Input: 1 x IEI iDP connector Display Output: 1 x VGA

Display IC: NXP - PTN3392 (DP to VGA)

Temperature: -10°C ~ 60°C Humidity: 5% ~ 95 non-condensing

Dimensions: 50 mm x 50 mm

Dimensions (mm)

| 1 x DP-VGA converter board | 1 x QIG |
| --- | --- |
| 1 x Internal DisplayPort cable, 450 mm length |  |

| Part No. | Description |
| --- | --- |
| DP-VGA-R10 | DisplayPort to VGA converter board (for IEI iDP connector), RoHS |

Packing List	Ordering Information

DisplayPort to DVI Converter Board (for IEI iDP Connector), RoHS

DP-DVI-R10

Features

- DisplayPort to DVI output

- Support up to 1920x1200 resolution

# Specifications

Display Input: 1 x IEI iDP connector Display Output: 1 x DVI

Display IC: Parade - PS161 (DP to HDMI / DVI)

Temperature: -10°C ~ 60°C Humidity: 5% ~ 95 non-condensing

Dimensions: 50 mm x 50 mm

Dimensions (mm)

LVDS-VGA-R10

Single-channel LVDS to VGA Converter Board (Support IEI Model Only)

Dimensions (mm)

36.07mm

12.50mm

Features

32.51mm

Packing List

| 1 x DP-DVI converter board | 1 x QIG |
| --- | --- |
| 1 x Internal DisplayPort cable, 450 mm length |  |

- Provide 2nd VGA option for dual VGA display

- Resolution adjusted by BIOS: Support 1024x768, 800x600, 640x480

- 18-bit LVDS to VGA converter board

LVDS24-01

24-bit dual-channel output

18-bit to 24-bit LVDS Converter Board

Dimensions (mm)

Features

- Resolution supports up to 1280x1024

- Support below configuration:

» Single-channel 18-bit to single-channel 24-bit

» Single-channel 18-bit to dual-channel 24-bit

18-bit single/

dual-channel input

* Note: The module does not support dual-channel 18-bit to single-channel 24-bit conversion. Ordering Information

» Dual-channel 18-bit to dual-channel 24-bit

Packing List

1 x LVDS to VGA converter board

1 x QIG

Power cable

P/N: 32100-130300-RS

Packing List

Ordering  Information

| Part No. | Description |
| --- | --- |
| DP-DVI-R10 | DisplayPort to DVI converter board (for IEI iDP connector), RoHS |

Ordering  Information

| Part No. | Description |
| --- | --- |
| LVDS-VGA-R10 | 18-bit LVDS to VGA converter board |
| 32600-112300-RS | 30-pin LVDS connecting cable |
| 32602-026400-100-RS | 20-pin LVDS connecting cable |

| Part No. | Description |
| --- | --- |
| LVDS24-01-R10 | 18-bit to 24-bit LVDS converter board |
| 32602-007900-100-RS | Single-channel 18-bit 20-pin to 24-bit 30-pin LVDS connection cable |
| 32602-008000-200-RS | Dual-channel 18-bit 30-pin to 24-bit 30-pin LVDS connection cable |

1 x 18-bit to 24-bit LVDS converter board

1 x Power cable

1 x QIG