w w w . i ei w or l d .c om

Digital Signage System

IDS-300-BW

# Features

- Fanless system with Intel® Celeron® N3160 processor

- Three HDMI ports

- Two full-size PCIe Mini slots for expansion

- Four USB 3.0 ports

- Two GbE LAN ports

HDMI

Fanless

Display

Triple

PCIe Mini Expansion

GbE

USB

USB 3.0

# Specifications

Fully Integrated I/O

Front View

Power Switch	Aduio

Rear View

| Model Name |  | IDS-300-BW |
| --- | --- | --- |
| Chassis | Color | Silver |
|  | Dimensions (WxDxH) (mm) | 241 x 160 x 39 |
|  | System Fan | Fanless |
|  | Chassis Construction | Aluminum alloys |
| Motherboard | CPU | Intel® Celeron® N3160 (up to 2.24GHz, quad-core, 2MB cache, TDP=6W) |
|  | Chipset | SoC |
|  | System Memory | 1 x 204-pin DDR3L SO-DIMM 4 GB pre-installed (system max: 8GB) |
| Storage | Hard Drive | 1 x 2.5'' SATA 6Gb/s HDD/SSD Bay |
| I/O Interfaces | USB 3.0 | 4 |
|  | Ethernet | 2 X RJ-45 PCIe GbE by RTL8111E controller |
|  | COM Port | 2 x RS-232 (RJ-45) |
|  | Button | 1 x Power button 1 x AT/ATX Switch |
|  | Display | 3 x HDMI |
|  | Resolution | HDMI 1&2 : up to 3840x2160 @ 30Hz HDMI 3 : up to 2560x1600 @ 60Hz |
|  | Audio | 1 x Line-in, 1 x Line-out, 1 x Mic-in |
|  | Wireless | 1x 802.11a/b/g/n/ac (optional) |
| Expansions | PCIe Mini | 1 x Full-size PCIe Mini* 1 x Full-size PCIe Mini (supports mSATA, colay with SATA) |
| Power | Power Input | DC Jack: 12 V DC |
|  | Power Consumption | 12 V@2.48 A (Intel® Celeron® N3160 with 4GB memory ) |
| Reliability | Mounting | Wall mount, VESA 100 |
|  | Operating Temperature | -10°C ~ 40°C with air flow (SSD), 10% ~ 95%, non-condensing |
|  | Operating Shock | Half-sine wave shock 5G, 11ms, 3 shocks per axis |
|  | Operating Vibration | MIL-STD-810F 514.5 C-1(HDD) |
|  | Weight (Net/Gross) | 1.8 kg/ 3kg |
|  | Safety/EMC | CE/FCC |
| OS | Supported OS | Microsoft® Windows® 8 Embedded, Microsoft® Windows® Embedded Standard 7 E |

DC

Input

AT/ATX

Mode

Clear CMOS

*IDS-300-BW-QGWN3/4G-R10 has only one PCIe mini slot

| Part No. | Description |
| --- | --- |
| IDS-300-BW-N3/4G-R10 | Fanless embedded system with Intel® Celeron N3160 1.6GHz, (up to 2.24 GHz, Quad Core, TDP 6W), pre-installed 4GB DDR3L memory, 12V DC, triple HDMI display, RoHS |
| IDS-300-BW-QGW- N3/4G-R10 | Fanless embedded system with Intel® Celeron N3160 1.6GHz, (up to 2.24 GHz, Quad Core, TDP 6W), pre-installed 4GB DDR3L memory, with QTS-Gateway, 12V DC, triple HDMI display, RoHS |

Ordering Information

3 x HDMI	2 x RS-232 4 x USB 3.0 2 x LAN

4-M4

100

256

240.70

IDS-300-BW

100

160

100

Dimensions (Unit: mm)

20

6 39

Options

| Part No. | Description |
| --- | --- |
| EMB-WIFI-KIT11-R10 | 1T1R Wi-Fi module kit for embedded system, IEEE802.11a/b/g/ n/ac Wi-Fi with Bluetooth 4.0/3.0+HS, 1 x Wi-Fi module, 2 x 250mm RF cable, 2 x Antenna, RoHS |
| IDS-300-WES7E64-R10 | OS Image with Windows® Embedded Standard 7 E 64-bit for IDS-300, DVD-ROM, RoHS |
| IDS-300-W10E64-E-R10 | OS Image with Windows 10 IoT Enterprise Entry 64-bit for IDS-300,W/DVD-ROM, RoHS |

Packing List

IDS-300-BW-2017-V12

Industrial System

| 1 x Utility CD | 1 x Power Adapter | 1 x Mounting Bracket |
| --- | --- | --- |