Vehicle Surveillance System

IVS-200-ULT2

mSATA

-20°C~60°C

Features

- Intel® i5-5350U dual-core 1.8 GHz CPU or Intel® Celeron® 3755U dual-core 1.7 GHz CPU

- Four Intel® GbE LAN ports

- 9 V~36 V DC input

- Two RS-232/422/485

- E-Mark certification

E

E-Mark

Four LAN

Fanless

9~36V

DV Input

Specifications

| Model Name |  | IVS-200-ULT2 |
| --- | --- | --- |
| Chassis | Color | Blue C |
|  | Dimensions (WxHxD) (mm) | 255 x 63 x 150 |
|  | System Fan | Fanless |
|  | Chassis Construction | Extruded aluminum alloy |
| Motherboard | CPU | Intel® Core™ i5-5350U Processor (dual-core, 1.8 GHz, 15W TDP) or Intel® Celeron® 3765U Processor (dual-core, 1.9 GHz, 15W TDP) |
|  | Chipset | SoC |
|  | System Memory | 2 x 204-pin DDR3L SO-DIMM slot 4 GB Pre-installed (system max. 16 GB) |
| Storage | Hard Drive | 1 x 2.5" SATA 6Gb/s HDD/SSD removable drive bay |
|  | mSATA | 1 x mSATA (SATA 6Gb/s) |
| Communication | WLAN | PCIe Mini Wi-Fi 802.11b/g/n (optional) |
|  | Bluetooth | Bluetooth 2.1 (combined with WWAN optional module) |
|  | WWAN | PCIe Mini WCDMA/HSDPA/HSUPA (optional) |
| I/O Interfaces | USB 3.1 Gen 1 | 2 |
|  | USB 2.0 | 2 |
|  | Ethernet | 4 x RJ-45 connector LAN1: PCIe GbE by Intel® I218 LAN2: PCIe GbE by Intel® I211 LAN3: PCIe GbE by Intel® I211 LAN4: PCIe GbE by Intel® I211 |
|  | COM Port | 2 x DB-9 RS-232/422/485 4 x DB-9 RS-232 (optional) |
|  | Display | 1 x VGA, 1 x HDMI |
|  | Resolution | VGA: up to 2560 x 1600 @ 60 Hz HDMI: up to 3840 x 2160 @ 60 Hz |
|  | DIO | 8 bit digital I/O (4-bit input, 4-bit output) (optional) |
|  | SIM | 1 x on board SIM card slot |
|  | Antenna | 1 x 3G, 1 x BT, 1 x Wi-Fi |
|  | Audio | 1 x Line out, 1 x Mic in |
| Expansions | PCIe Mini | 1 x Full-size PCIe Mini (reserved for WWAN or mSATA)* 1 x Full-size PCIe Mini (reserved for WLAN) |
| Power | Power Input | DC jack : 9 V~36 V DC |
|  | Consumption | 12 V @ 2.1 A (Intel® i5-5350U with 4 GB DDR3 memory) |
| Reliability | Mounting | VESA 100 |
|  | Operating Temperature | -20°C ~ 60°C with air flow (SSD) |
|  | Storage Temperature | -30°C ~ 70°C |
|  | Humidity | 10% ~ 95%, non-condensing |
|  | Operating Shock | Half-sine wave shock 5G, 11ms, 3 shocks per axis |
|  | Operating Vibration | MIL-STD-810F 514.5C-2 (with SSD) |
|  | Weight (Net/Gross) | 2.2 kg/3 kg |
|  | Safety/EMC | CE/FCC/E-Mark |
| OS | Supported OS | Microsoft® Windows® Embedded 8, Microsoft® Windows® Embedded Standard 7 E |

*IVS-200-ULT2-QGW-R10 has only one PCIe mini slot(Full size)

# Fully Integrated I/O

Front view	Rear view

LED Indicators:

HDD, Power	COM 1	HDMI	COM 2

2 x USB 3.1

Gen 1	ACC Mode	Reset

ATX/AT

Mode	2 x USB 2.0

Power Switch

2.5" HDD/SSD Drive Bay

Audio

Power In (Terminal Block)

4 x LAN

VGA

Dimensions (Unit: mm)

IVS-200-ULT2

# Ordering Information

| Part No. | Description |
| --- | --- |
| IVS-200-ULT2-i5/4G-R10 | Fanless system with Intel® Core™ i5-5350U CPU, up to 2.9GHz, TDP 15W, four GbE LAN ports, one HDMI, one VGA, two RS-232/422/485, one 2.5" removable SATA HDD bay, 4 GB memory pre-installed, 9 V~36 V DC, R10, RoHS |
| IVS-200-ULT2-C/4G-R10 | Fanless system with Intel® Celeron® 3765U 1.7GHz CPU, TDP 15W, four GbE LAN ports, one HDMI, one VGA, two RS-232/422/485, one 2.5" removable SATA HDD bay, 4 GB memory pre-installed, 9 V~36 V DC, R10, RoHS |
| IVS-200-ULT2-QGW-R10 | Fanless System with Intel® Core™ i5-5350U up to 2.9GHz, TDP 15W, 4 x GbE LAN port , 1 x HDMI, 1 x VGA, 2 x RS-232/422/485, 1 x 2.5’’ removable SATA HDD bay, 8G memory pre-installed, with QTS-Gateway, 9~36V DC, RoHS |

Options

| Item | Part No. | Description |
| --- | --- | --- |
| OS | IVS-200-ULT2-WES7E64-R10 | OS Image with Windows® Embedded Standard 7 E 64-bit for IVS-200-ULT2, DVD-ROM, RoHS |
| Serial cable | 32205-005400-100-RS | RS-232 cable, 150 mm, 2x5 pin |
| DIO cable | 32231-000100-200-RS | DIO cable, 150 mm, 2x5 pin |

Packing List

| Item | Part No. | Qty | Description |
| --- | --- | --- | --- |
| Screw pack | 44013-030041-RS | 4 | Screw pack |
|  | 44005-030083-RS | 4 | Screw pack |
| Wall Mount Kit | 41020-0433C2-00-RS | 2 | Wall mount bracket |